Department of Computer Engineering

TE Computer-B (2024-25 Sem I)

Computer Networks and Security

CNS Simulation Assignment 1: VLAN Configuration

[CO1-CO2, BT: L3 (Apply)] [Max Marks: 10]

Date of Assignment 11th July 2025			Last Date of Submission: 21st July 2025

Demonstrate of Virtual LAN (VLAN) using Packet Tracer

Objective: To configure and demonstrate Virtual LANs (VLANs) using Cisco Packet Tracer by creating two VLANs and testing inter-VLAN communication through a router.

IP Address Series: 192.168.1.1 to 192.168.1.254 for VLAN1 (Device 1 and 2)

192.168.47.1 to 192.168.47.254 for VLAN2 (Device 3 and 4)

Steps Involved:

- Network Topology: 1 Router, 1 Switch, 4 PCs (2 in VLAN1, 2 in VLAN47),  Cabling

- IP Address Configuration of PCs:

Device 1: IP – 192.168.1.2, Subnet Mask – 255.255.255.0, Gateway – 192.168.1.1

Device 2: IP – 192.168.1.3, Subnet Mask – 255.255.255.0, Gateway – 192.168.1.1

Device 3: IP – 192.168.47.2, Subnet Mask – 255.255.255.0, Gateway – 192.168.47.1

Device 4: IP – 192.168.47.3, Subnet Mask – 255.255.255.0, Gateway – 192.168.47.1

- Configuration of VLAN Database at Switch:

- Go to Switch0 > Config > VLAN Database, enter VLAN Number 47, VLAN Name RohitSharma, and click Add.

- Select GigaEthernet0/3, set mode to Access, and assign it to VLAN 47: RohitSharma from the dropdown.

4.Configuration of router

- Go to Router CLI, enter: enable → configure terminal

- Create sub interface for VLAN 1: interface GigabitEthernet0/0.1 → encapsulation dot1Q 1 → ip address 192.168.1.1 255.255.255.0 → exit

- Create sub interface for VLAN 47: interface GigabitEthernet0/0.47 → encapsulation dot1Q 47 → ip address 192.168.47.1 255.255.255.0 → exit

- Activate main interface: interface GigabitEthernet0/0 → no shutdown → exit

5.Ping Testing:

- On PC1, open Command Prompt and ping PC2 (same VLAN): ping 192.168.1.3 — Successful reply received.

- On PC1, ping the Router sub interface for VLAN 47: ping 192.168.47.1 — Successful, confirming router reachability.

- On PC2, ping PC1 (same VLAN): ping 192.168.1.2 — Successful reply.

- On PC2, ping PC3 and PC4 (different VLAN): ping 192.168.47.3 and ping 192.168.47.2 — Both successful, confirming inter-VLAN routing.

6.Real Mode Simulation and Event Simulation:

- At the bottom-right, click Simulation Mode.

- Now click the “Add Simple PDU” (envelope icon) from the bottom-left toolbar.

- Click on Device 1 (192.168.1.2), then click on Device 2 (192.168.1.3).

- This drops a simulated ping (ICMP packet) between the two devices.

- After a few steps, the simulation should complete and show packet delivery. Message changes to: Last Status: Successful | Source: Device 1 | Destination: Device 2 | Type: ICMP

- At the bottom-right, click the Simulation tab.

- Click the Add Simple PDU tool (envelope icon in the bottom-left toolset).

- First, click on Device 1 (192.168.1.2), then click on Device 2 (192.168.1.3) to create a simulated ping.

- Press the Capture/Forward button to step through the packet's path.

- Green dots/arrows will appear showing the packet flow from source to destination.

- We will see events updating live in the Event List Panel (right side).